Network Design Principles

Topic: Simple Network Design with Cisco Packet Tracer

# Overview

In this in-class activity, you will create a basic local network in Cisco Packet Tracer. You’ll practice configuring devices, assigning IP addresses, and verifying connectivity—all without subnets or VLANs. If you’re looking for a more advanced challenge, check out the Bonus Task at the end.

# Part 1: Planning Your Network

- Scenario & Requirements

  - You have a single router connected to an ISP.

  - One switch is placed in your local network.

  - You have 3–4 PCs that need to communicate with each other and have internet access.

  - The entire network will use one IP range (e.g., 192.168.1.0/24).

- Basic Topology

  - Use a star topology: The router connects to the switch, and all PCs connect to the switch.

  - Assign IP addresses in the 192.168.1.x range for each PC, ensuring they have the same subnet mask (e.g., 255.255.255.0).

  - Set the default gateway on each PC to the router’s LAN interface IP.

- Documentation

  - Quickly sketch or note the layout: router → switch → PCs.

  - Record each PC’s IP address and the router’s IP.

# Part 2: Building the Network in Cisco Packet Tracer

First, let's get all the necessary components into our digital workspace.

- Open Cisco Packet Tracer. You will see a large white area—this is your workspace where you will build the network.

- Select and Place a Router.

  - In the bottom-left corner, click on the Network Devices icon (it looks like a router).

  - Just below that, click on the Routers sub-category.

  - Find the 4331 router model. Click on it.

  - Move your mouse to the top of the white workspace and click again to place the router.

- Select and Place a Switch.

  - Stay in the Network Devices category, but now click the Switches sub-category.

  - Find the 2960 switch model. Click on it.

  - Click in the workspace below your router to place the switch.

- Select and Place PCs.

  - In the bottom-left corner, click on the End Devices icon (it looks like a computer).

  - The PC icon will be the first one. Click on it.

  - Click in the workspace below the switch to place your first PC.

  - Repeat this two more times, placing a total of 3 PCs in a row.

Your workspace should now have one 4331 router, one 2960 switch, and three PCs laid out.

### Part 2: Connecting Your Devices with Cables 🔌

Now we'll connect the devices together using virtual cables to form a star topology.

- Select the Cable Type.

  - In the bottom-left menu, click the Connections icon (it looks like a lightning bolt).

  - Find the Copper Straight-Through cable. It's the icon with a solid black line. Click on it.

- Connect the Router to the Switch.

  - Click on your Router. A menu of available ports will appear.

  - Select GigabitEthernet0/0/0.

  - Now, move your mouse over the Switch and click on it.

  - From the switch's port menu, select GigabitEthernet0/1. A line will now connect the two devices.

- Connect the PCs to the Switch.

  - Make sure the Copper Straight-Through cable is still selected.

  - Click on the first PC (PC0) and select its only port, FastEthernet0.

  - Click on the Switch and select port FastEthernet0/1.

  - Repeat this for the other two PCs:

    - Connect PC1 (FastEthernet0) to the Switch (FastEthernet0/2).

    - Connect PC2 (FastEthernet0) to the Switch (FastEthernet0/3).

Your topology is now complete! It should show the router at the top, connected to the switch, which is then connected to all three PCs.

### Part 3: Configuring the IP Addresses ⚙️

Devices need IP addresses to communicate. We will configure the router first, as it will serve as the "Default Gateway" for all the PCs.

#### Step 3.1: Configure the Router

We will use the Command Line Interface (CLI) to configure the router, as this is a common practice in real-world networking.

- Open the Router's CLI.

  - Click on the Router in your workspace.

  - A new window will open. Click on the CLI tab.

  - You will see some text scroll by. It will end with the question: Would you like to enter the initial configuration dialog? [yes/no]:. Type no and press Enter.

  - Press Enter one more time to get to the Router> prompt.

- Enter the Configuration Commands.

  - Type enable and press Enter. Your prompt will change to Router#. This is Privileged Mode.

  - Type configure terminal and press Enter. The prompt becomes Router(config)#. This is Global Configuration Mode, where you can make changes.

  - Type interface GigabitEthernet0/0/0 and press Enter. This command selects the specific port on the router that you connected to the switch. The prompt will change to Router(config-if)#.

  - Now, assign the IP address and subnet mask. Type ip address 192.168.1.1 255.255.255.0 and press Enter. This sets the router's address to 192.168.1.1.

  - Finally, turn the port on. By default, router ports are shut down. Type no shutdown and press Enter. You will see messages indicating the state has changed to "up".

  - exType exit and press Enter to leave the interface configuration.

  - Type exit again and press Enter to leave the global configuration.

You have successfully configured the router! You can close the router's configuration window. Notice the small link light on the cable connection between the router and switch should now be green.

#### Step 3.2: Configure the PCs

Now, let's give each PC a unique IP address within the same network.

- Configure the First PC (PC0).

  - Click on PC0 in the workspace.

  - Go to the Desktop tab.

  - Click the IP Configuration icon.

  - Make sure the Static option is selected.

  - Fill in the following details:

    - IPv4 Address: 192.168.1.10

    - Subnet Mask: 255.255.255.0 (this should fill in automatically when you click it)

    - Default Gateway: 192.168.1.1 (this is the router's IP address)

  - Close the IP Configuration window.

- Configure the Second PC (PC1).

  - Click on PC1, go to Desktop > IP Configuration.

  - Fill in the details with a unique IP:

    - IPv4 Address: 192.168.1.11

    - Subnet Mask: 255.255.255.0

    - Default Gateway: 192.168.1.1

- Configure the Third PC (PC2).

  - Click on PC2, go to Desktop > IP Configuration.

  - Fill in the details with another unique IP:

    - IPv4 Address: 192.168.1.12

    - Subnet Mask: 255.255.255.0

    - Default Gateway: 192.168.1.1

All devices are now configured! All the link lights on your connections should be green.

# Part 3: Testing & Troubleshooting

The final step is to test if your network works by sending ping requests. A ping is a simple test to see if one device can reach another.

- Test PC to Router Connectivity.

  - Click on any PC, for example, PC0.

  - Go to the Desktop tab and open the Command Prompt.

  - In the command prompt, type ping 192.168.1.1 and press Enter.

  - You should see four "Reply from..." messages. This confirms your PC can communicate with the router.

Test PC to PC Connectivity.

  - In the same command prompt on PC0, let's ping PC1.

  - Type ping 192.168.1.11 and press Enter.

  - Again, you should see successful replies. This confirms that devices on your local network can communicate with each other through the switch.

## Part 4: Analysis & Reflection

- Group Discussion

  - Did you encounter any configuration errors?

  - How did you fix them?

- Practical Considerations

  - If you had more PCs, how would you maintain organisation?

  - In a real office, what security measures might you add beyond a simple password?

- Written Summary (Optional)

  - Summarize your network setup and any lessons learned.

# Deliverables

- Packet Tracer File (.pkt) containing your simple network.

- Diagram or screenshot of your setup.

- Brief Reflection on challenges and takeaways.

# Tips

- Time Allocation: About 50 minutes for setup, 10 minutes for testing.

- Collaboration: Work in small teams; each person can configure one PC.

- Guidance: Focus on basic device configuration and ensuring correct IP settings.

# Bonus Task: Sub-netted Network (Advanced Challenge)

If you’ve found the basic setup straightforward and you’d like to go further, try creating a slightly more complex network that uses subnets:

- Objective: Divide your network into two networks (e.g., 192.168.10.0/24 and 192.168.20.0/24) connected by a single router.

- Topology:

  - Use a router with two LAN interfaces (or sub-interfaces with Router-on-a-Stick if you’re comfortable). Each interface corresponds to a unique subnet.

  - Connect each LAN interface to a separate switch.

  - Place 2 PCs in each subnet.

- IP Configuration:

  - Assign the router interface for Subnet 1 as 192.168.10.1/24 and Subnet 2 as 192.168.20.1/24.

  - PCs in Subnet 1 use IPs in the 192.168.10.x range, with the gateway 192.168.10.1.

  - PCs in Subnet 2 use IPs in the 192.168.20.x range, with the gateway 192.168.20.1.

- Verification:

  - Ping across subnets (e.g., from a PC in Subnet 1 to a PC in Subnet 2). If it fails, double-check IP, gateway, and router settings.

  - Optionally set up RIPv2 or static routes if needed (though Packet Tracer often routes automatically when each interface has a unique network).

- Reasoning:

  - This simulates a real scenario where different departments or areas have their own IP ranges.

# Deliverables (Bonus):

- A .pkt file or an extension of your existing one showing the two-subnet design.

- A short note on your IP plan and any issues you solved.

# Appendix: Explanation of Device Choices

When building a network in a simulator like Cisco Packet Tracer, you have many device models to choose from. For the beginner lab, the models were selected because they are common, versatile, and perfectly suited for the task without being overly complex. Here is the reasoning behind each choice.

#### Why the 4331 Router?

- Industry Relevance: The Cisco 4331 is an Integrated Services Router (ISR). This type of router is widely used in real-world small-to-medium-sized business branch offices. Learning on a device you might actually encounter is a valuable experience.

- Appropriate Features: It comes with Gigabit Ethernet interfaces, which are the modern standard for wired network speeds. For this lab, we only need one of these interfaces to connect to our local network switch. It has all the basic routing capabilities needed and supports more advanced features you might learn later, making it a great all-around choice for educational purposes.

- Simplicity for the Task: While powerful, its basic configuration for a simple network is straightforward. Using the Command Line Interface (CLI) on this router is a fundamental skill, and the steps are clear and repeatable.

#### Why the 2960 Switch?

- The Quintessential "Learning" Switch: The Cisco Catalyst 2960 is one of the most popular Layer 2 switches used in networking education and certification training (like the CCNA). Its presence in Packet Tracer is specifically for labs like this one.

- Sufficient Port Density: The 2960 model in Packet Tracer has 24 FastEthernet ports and 2 GigabitEthernet ports. The lab requires connecting 3-4 PCs and one router, so this switch provides more than enough ports for the task and for potential expansion.

- Focus on the Core Function: As a Layer 2 switch, its primary job is to forward traffic between devices on the same network, which is exactly what's needed to allow the PCs to communicate with each other. The lab explicitly avoids complex switch features like VLANs, and the 2960's default configuration is perfect for this simple setup.

#### Why the "PC" End Device?

- Essential Testing Tools: The generic "PC" in Packet Tracer is the ideal end device because it includes the two critical software tools needed for this lab:

  - IP Configuration: A simple graphical menu to set the PC's static IP address, subnet mask, and default gateway.

  - Command Prompt: An essential tool for running the ping command to test connectivity to the router and to other PCs, which is a core requirement of the lab.

- Directly Represents the Requirement: The lab scenario calls for several PCs that need network access, and this device is the direct simulation of that.

#### Why the Copper Straight-Through Cable?

- Adherence to Cabling Standards: In networking, the type of cable you use matters. The rule of thumb is:

  - Use a Straight-Through cable to connect devices that are different from each other (e.g., a PC to a Switch, or a Switch to a Router).

  - Use a Crossover cable to connect devices that are similar (e.g., a Switch to a Switch, or a PC to a PC).

- Correct for the Topology: In our lab, we make two types of connections: Router-to-Switch and PC-to-Switch. Both of these are connections between different types of devices, making the Straight-Through cable the correct technical choice. While Packet Tracer's "auto-select" feature would also work, manually selecting the correct cable reinforces proper networking practices.